END SEMESTER PRACTICAL EXAMINATIONS UG –APRIL / MAY 2026

DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

Course & Class:   B.E. ECE						Semester: IV

Staff Name: S. Alwyn Rajiv			 			 Venue: ECE Lab I

Subject Code & Name: EC2257 - Linear Integrated Circuits Laboratory

Date & Session: 04.05.2024 (FN & AN)

RUBRICS

| Aim/ Apparatus required / Procedure (20) | Design & Circuit Diagram (50) | Verification & Results (20) | Viva-Voce (10) | Total (100) |
| --- | --- | --- | --- | --- |

QUESTION PAPER

| Q.No. | Question | Mark | Assign to No. of Students |
| --- | --- | --- | --- |
|  | - (i) An Inverting amplifier is to amplify a 75mV signal to a level of 5V. Using IC 741 op-amp, design a suitable circuit e2=1+xy+abc+xya (ii) Simulate the Astable multivibrator using OPAMP IC741 using PSPICE. | 50 50 | 2 |
|  | (i) The non-inverting amplifier with a ±15V supply is to produce maximum possible output voltage and is to have a voltage gain of 3.3. Design and construct a suitable circuit using IC 741. (ii) Simulate the Monostable multivibrator using OPAMP IC741 using PSPICE. | 50 50 | 2 |
|  | (i) Design a Differential amplifier for a gain of 10 using op-amp IC 741. (ii) Simulate the active wide band pass filter for a cut off frequency of 450 kHz and 2 kHz and pass band gain of 5 with IC 741 using PSPICE. | 50 50 | 2 |
|  | Design and construct an instrumentation amplifier using operational amplifier IC 741 and verify its operation. | 10 | 1 |
|  | (i) Design a differentiating circuit to give an output of 5V when the input voltage changes by 1V in the time of 100µsec. Use an op-amp to construct the circuit and assume the input waveform to be sine and square wave. (ii) Simulate the high pass filter at a cut off frequency of 3 kHz and pass band gain of 5. Plot the frequency response in PSPICE. | 50 50 | 2 |
|  | (i) Use of an op-amp, design an integrating circuit to produce a triangular wave output with peak-to-peak amplitude of 5V, when the input is a square wave with an amplitude of ±3V and frequency of 1 kHz. (ii) Simulate the first order low pass and high pass filter to a cut off frequency of 3kHz using OP- amp in PSPICE. | 50 50 | 2 |
|  | (i) Design a first order high-pass Butterworth filter to have a cut-off frequency of 11 kHz. Use a 741 op-amp and estimate the highest signal frequency that will be passed. (ii) Simulate the inverting amplifier using PSPICE. | 50 50 | 2 |
|  | (i) Design and construct a free running multivibrator using op-amp with a frequency of 1kHz. (ii) Simulate the non-inverting amplifier using PSPICE. | 50 50 | 2 |
|  | (i) With a suitable circuit component for the value of UTP=+3V and LTP = -3V, design a Schmitt trigger circuit using IC 741 op-amp with Vcc= ±9V (ii) Simulate the free running oscillator to generate the square waveform of 50% duty cycle using NE555 in PSPICE. | 50 50 | 1 |
|  | (i) Design and construct the phase shift oscillator to oscillate at 300 Hz and plot the waveform using IC 741. (ii) Simulate the narrow band pass filter whose frequency response lying between 50 Hz to 250kHz in PSPICE. | 50 50 | 2 |
|  | (i) Design a Wien bridge oscillator to have an output frequency of 12 KHz. Use a op-amp with a supply voltage of ±15V. (ii) Simulate Digital to Analog converter using PSPICE. | 50 50 | 2 |
|  | (i) Using the LM723 IC regulator, design and construct the positive voltage regulator to produce an output voltage adjustable from 8 to 16 V. Determine the maximum output current that can be supplied. (ii) Simulate the Astable multivibrator using IC555 timer using PSPICE. | 50 50 | 1 |
|  | (i) Using LM317 construct the circuit to find the line and load regulation. (ii) Simulate the Monostable multivibrator using IC555 timer using PSPICE. | 50 50 | 2 |
|  | (i) Design and construct a low frequency oscillator which has a ladder type RC feedback circuit using operational amplifier IC 741. | 100 | 2 |
|  | (i) Design and construct a monostable multivibrator using NE555 timer. (ii) Simulate an active band pass filter with IC 741 using PSPICE. | 50 50 | 1 |
|  | (i) Design and construct a free running oscillator that generates square waveform using NE 555 timer. (ii) Simulate Digital to Analog converter using PSPICE | 50 50 | 2 |
|  | (i) Design and construct a circuit that produces an output voltage which is proportional to the time integral of the input voltage using operational amplifier IC 741. (ii) Simulate the Astable multivibrator using NE555 timer using PSPICE. | 50 50 | 2 |
|  | (i) Design and construct a circuit that produces the derivative of input voltage as output using operational amplifier IC 741. (ii) Simulate an active high pass filter for a cut-off frequency of 1000 Hz with IC 741 using PSPICE. | 50 50 | 2 |
|  | (i) Design an inverting amplifier for a gain of 10 using operational amplifier IC 741. (ii) Simulate an active low pass filter for a cut-off frequency of 1000 Hz with IC 741 using PSPICE. | 50 50 | 2 |
|  | (i) Design and construct a circuit that produces an output voltage which is proportional to the time integral of the input voltage using operational amplifier IC 741. (ii) Simulate a free running multivibrator that generates square waveform with frequency of 1 kHz with NE 555 using PSPICE. | 50 50 | 2 |
|  | (i) Configure Op-amp 741 as an inverting amplifier with R1=1kΩ and Rf=10kΩ. Calculate the loop gain. (ii) Simulate the Astable multivibrator using opamp NE555 timer using PSPICE. | 50 50 | 1 |
|  | i) Construct a noninverting amplifier using an Op-amp 741 with R1=1kΩ and Rf =10kΩ. (ii) Simulate the Monostable multivibrator using opamp NE555 timer using PSPICE. | 50 50 | 2 |
|  | (i) Assemble a differential amplifier circuit using Op-amp 741.Calculate the differential mode gain. (ii) Simulate the active wide band pass filter for a cut off frequency of 450kHz and 2 kHz and pass band gain of 5 with IC 741 using PSPICE. | 50 50 | 2 |
|  | (i) Construct a practical integrator circuit using Op-amp 741 to process the input square waveform signal with fs=1kHz and amplitude 100mVp-p. (ii) Simulate the high pass filter at a cut off frequency of 3 KHz and pass band gain of 5. Plot the frequency response in PSPICE. | 50 50 | 2 |
|  | (i) Identify a circuit that differentiate a input sinusoidal signal of fs=500Hz and amplitude 5Vp-p using Op-amp 741. (ii) Simulate the first order low pass and high pass filter to a cut off frequency of 3kHz using OP- amp in PSPICE. | 50 50 | 2 |
|  | (i) Design and implement a high pass filter at a cutoff frequency fL= 2kHz with pass band gain of 2. (ii) Simulate the inverting amplifier using PSPICE. | 50 50 | 2 |
|  | (i) Design and implement a Triangular wave generator for the frequency of 500Hz. | 100 | 2 |
|  | - Design and implement a phase shift oscillator circuit using IC 741 with fo=500Hz - Simulate the free running oscillator to generate the square waveform of 50% duty cycle using IC 555 in PSPICE. | 50 50 | 2 |
|  | (i) Construct a free running multivibrator using timer NE555 with duty cycle of 66.6%. (ii) Simulate the narrow band pass filter whose frequency response lying between 50 Hz to 250kHz in Design a triangular wave generator for the frequency of 500Hz. | 50 50 | 2 |
|  | (i) Design an adjustable voltage regulator using LM 317 with the following specifications: Output voltage vo=10V. (ii) Simulate Digital to Analog converter using PSPICE. | 50 50 | 2 |